Listing 3. A DTMF Decoder

#include <stdio.h>

#include <stdlib.h>

#include "portaudio.h"

#include <gsl/gsl_fft.h>

#define SAMPLE_RATE  (8192)

#define NUM_SECONDS     (1)

#define NUM_CHANNELS    (1)

#define DITHER_FLAG     (0)

#define FRAMES_PER_BUFFER  (1024)

#define THRESHOLD 10.0  // minimum "strength" to notice

#define PA_SAMPLE_TYPE  paFloat32

typedef float SAMPLE;

#define SAMPLE_SILENCE  (0.0f)

// We will hold 16 buffers of data in a ring

// so we can get behind a few buffers on processing

#define NRBUF 16

typedef struct

{

int         next;

int         first;

SAMPLE      *recordedSamples[NRBUF];

}

paTestData;

/* Called when touch tone detected */

// to avoid "runt errors" we need to see a digit twice in a row

// which means we need at least 1/4 second tone (this is not

// in spec for DTMF, by the way)

void detected(char digit)

{

// make sure we get two of each digit and then reject

// the rest (until we detect a gap)

static int lastdigit=0;

static int lastput=0;

if (digit&&digit!=lastput&&digit==lastdigit)

{

putchar(digit);

lastput=digit;

fflush(stdout);

}

if (!digit) lastput=0;

lastdigit=digit;

}

/* This routine will be called by the PortAudio engine when audio is needed.

** It may be called at interrupt level on some machines so don't do anything

** that could mess up the system like calling malloc() or free().

*/

static int recordCallback( void *inputBuffer, void *outputBuffer,

unsigned long framesPerBuffer,

PaTimestamp outTime, void *userData )

{

// Get data structure

paTestData *data = (paTestData*)userData;

SAMPLE *rptr = (SAMPLE*)inputBuffer;

SAMPLE *wptr = data->recordedSamples[data->next];

long framesToRecord;

long i;

// pick up next buffer and rotate

if (++data->next==NRBUF) data->next=0;

(void) outputBuffer; /* Prevent unused variable warnings. */

(void) outTime;

// Silence?

if( inputBuffer == NULL )

{

for( i=0; i<FRAMES_PER_BUFFER; i++ )

{

*wptr++ = SAMPLE_SILENCE;

}

}

else  // no, real data

{

for( i=0; i<FRAMES_PER_BUFFER; i++ )

{

*wptr++ = *rptr++;

}

}

return 0;  // don't terminate

}

// This is an FFT wrapper that uses the

// GNU Scientific Library (GSL) FFT function

void fft(unsigned N, SAMPLE datareal[], int binnums[], SAMPLE  bins[] )

{

// GSL wants an array of doubles not floats

static double *ary=NULL;

if (!ary) ary=malloc(sizeof(double)*N);  // malloc first time through

unsigned i;

if (!ary) return;

for (i=0;i<N;i++) ary[i]=datareal[i];

// Do FFT

gsl_fft_real_radix2_transform(ary,1,N);

for (i=0;i<8;i++)

{

int bin=binnums[i];

// find magnitude (N-bin) is imaginary part

bins[i]=sqrt(ary[bin]*ary[bin]+ary[N-bin]*ary[N-bin]);

// look at next bin too and average

bin++;

bins[i]+=sqrt(ary[bin]*ary[bin]+ary[N-bin]*ary[N-bin]);

bins[i]/2; // average

}

// The above code only looks at the bins we ask (plus the next bin)

// which is more efficient than looking at everything

// If you want to copy all the outputs

//  for (i=0;i<N/2;i++)

//    {

//    data[i]=ary[i];

//    dataimg[i]=ary[N-i];

//    }

// this only copied the meaningful half

// don't worry about the rest

// don't free ary because we'll keep using it

}

/*******************************************************************/

// Note that if you don't supply an argument, the code gets the

// default audio input device. If you supply a number, you get that

// device. Portaudio numbers devices 0, 1, 2,... If you have multiple

// sound devices, you can use tools provided with Portaudio to dump

// the sound card numbers, but the default should be your default

// soundcard

// second argument, if present is the threshold

int main(int argc, char *argv[])

{

PortAudioStream *stream;

PaError    err;

paTestData data;

int        i;

int        totalFrames;

int        numSamples;

int        numBytes;

float threshold=THRESHOLD;

SAMPLE     max, average, val;

// The bins that hold DTMF data

int binnums[]= { 87, 96, 106, 117, 151, 167, 184, 204 };

SAMPLE bins[8];

char matrix[4][4]= { { '1', '2', '3', 'A' },

{'4', '5', '6',  'B' },

{ '7', '8', '9', 'C' },

{ '*', '0', '#', 'D' }};

SAMPLE lowv,highv;

int low,high;

data.first=0;

data.next=0;

if (argc>=3) threshold=atof(argv[2]);  // custom threshold

numSamples = FRAMES_PER_BUFFER*NUM_CHANNELS;

numBytes = numSamples * sizeof(SAMPLE);

for (i=0;i<NRBUF;i++)

{

data.recordedSamples[i] = (SAMPLE *) malloc( numBytes );

}

if( data.recordedSamples == NULL )

{

printf("Could not allocate record array.\n");

exit(1);

}

// start portaudio

err = Pa_Initialize();

if( err != paNoError ) goto error;

/* Record some audio. -------------------------------------------- */

err = Pa_OpenStream(

&stream,

argc==1?Pa_GetDefaultInputDeviceID():atoi(argv[1]),  // device

NUM_CHANNELS,

PA_SAMPLE_TYPE,

NULL,

paNoDevice,  // no playback required here

0,

PA_SAMPLE_TYPE,

NULL,

SAMPLE_RATE,

FRAMES_PER_BUFFER,            /* frames per buffer */

0,               /* number of buffers, if zero then use default minimum */

0, /* paDitherOff, // flags */

recordCallback,

&data );

if( err != paNoError ) goto error;

err = Pa_StartStream( stream );

if( err != paNoError ) goto error;

printf("Listening! (threshold=%f)\n",threshold);

fflush(stdout);

// This loop will run forever, processing audio data

while( Pa_StreamActive( stream ) )

{

while (data.next==data.first) Pa_Sleep(1);

fft(FRAMES_PER_BUFFER,data.recordedSamples[data.first],binnums,bins);

if (++data.first==NRBUF) data.first=0;

// So let's see what we got

// each bin  is worth 8hz so

// 697hz - bin 87   (all rounded down)

// 770hz - bin 96

// 852hz - bin 106

// 941hz - bin 117

// 1209hz - bin 151

// 1336hz - bin 167

// 1477hz - bin 184

// 1633hz - bin 204

// so taking the average of two adjacent bins will

// give us a good idea

low=high=-1;

lowv=highv=-1.0;

// Find magnitude of results we are interested in

for (i=0;i<8;i++)

{

if (i<4)

{

if (bins[i]>lowv && bins[i]>=THRESHOLD)

{

lowv=bins[i];

low=i;

}

}

else

{

if (bins[i]>highv && bins[i]>=THRESHOLD)

{

highv=bins[i];

high=i-4;

}

}

}

// check to see that we found something

// ought to check that highv and lowv are not too different

if (low>=0 && high>=0)

detected(matrix[low][high]);

else

detected(0);  // tell detector no input found

}  // end processing loop

// Should never get here because callback never terminates

// so Pa_StreamActive is always true

err = Pa_CloseStream( stream );

if( err != paNoError ) goto error;

free( data.recordedSamples );

Pa_Terminate();

return 0;

error:

Pa_Terminate();

fprintf( stderr, "An error occurred while using the portaudio stream\n" );

fprintf( stderr, "Error number: %d\n", err );

fprintf( stderr, "Error message: %s\n", Pa_GetErrorText( err ) );

return -1;

}